# SolarInvest: Methodology Report

AI-Powered Residential PV Sizing

# 1. Problem Formulation and Core Formulation

## 1.1 Problem Statement

The system addresses the residential solar sizing problem: given a homeowner's location, budget, roof dimensions, and household profile, recommend a PV system (panel count, brand, and optional battery) that balances technical feasibility, budget constraints, and financial returns.

The decision variables are: N (panel count), Panel brand (from a catalog of 9 manufacturers), and Battery (add / evaluate later / PV-only). Subject to: Roof capacity (N ≤ N_roof), Budget (CAPEX ≤ budget_usd), and User brand preference (if specified).

## 1.2 Core Formulation

The model produces two scenarios. Optimal scenario (technically best): N_opt = min(N_100%, N_roof) where N_100% is panels for 100% annual consumption offset. Recommended scenario (budget-aware): N_rec = min(N_70%, N_budget, N_roof) where N_70% targets 70% offset and N_budget is max panels affordable at the given budget.

The financial objective is 10-year NPV: NPV = -Net_CAPEX + Σ(Savings_y / (1+r)^y) with r = 7% discount rate. Net CAPEX is post-ITC (30% federal credit).

# 2. Input Data: Extraction, Processing, and Modelling

## 2.1 Weather Data

Source: Open-Meteo Archive API. Variables: daily (temperature max/min, shortwave radiation sum), hourly (cloud cover, shortwave radiation). Processing: weather_fetcher.py fetches 1–5 years of history, aggregates to weekly (weekly_max_temperature, weekly_avg_irradiance, weekly_avg_cloud_cover). fetch_irradiance_annual() derives mean annual GHI (kWh/m²/yr). Fallback: 2080 kWh/m²/yr.

## 2.2 Household Usage Data

Source: EIA San Diego regional load (San_Diego_Load_EIA_Fixed.csv), hourly MW. Processing: avg_kw = MW_Load × 1000 / SDGE_TOTAL_CUSTOMERS (1,040,149 customers). Nine variability factors applied: F1 longitude (coastal→inland 0.85×–1.25×), F2 latitude (0.90×–1.10×), F3 elevation, F4 household size, F5 urban density (0.7×–1.3×), F6 economic proxy, F7 solar adoption, F8 EV charging (7.2 kW per EV), F9 multi-generational. Deterministic via SHA-256 hash of (lat, lon).

## 2.3 Tariff Modelling

Source: SDG&E TOU rate CSVs (tou_dr_daily_2021_2025.csv). Periods: On-peak 16:00–21:00, Super-off-peak 00:00–06:00 and Mar–Apr 10:00–14:00, Off-peak otherwise. build_hourly_tariffs() maps each of 8760 hours to the correct rate.

# 3. Feature Engineering

feature_engineering.py derives ~70 features in seven categories: (1) Electricity: load distribution, seasonality, growth, peak load. (2) Weather/Solar: irradiance stats, PV efficiency, consumption–irradiance correlation. (3) Household: kWh per occupant, cost, annual spend. (4) Cross-dataset: panels for 100%/70%/50% offset, break-even, NPV, IRR, ROI, nighttime load ratio. (5) Risk: price and irradiance sensitivity. (6) EV & Budget: EV charging load, budget analysis. (7) format_for_llm() produces a structured text block for the prompt.

# 4. Mathematical Optimisation

## 4.1 Constants

| Constant | Value | Source |
| --- | --- | --- |
| G_ref | 1000 W/m² | STC |
| PR (performance ratio) | 0.80 | pv_tools.py |
| Federal ITC | 30% | config.yaml |
| Utility escalation | 6%/yr | pv_tools.py |
| Discount rate | 7% | pv_tools.py |
| O&M | $0.005/W/yr | pv_tools.py |
| NEM export credit | $0.10/kWh | pv_tools.py |
| Inverter replacement | $2000 at year 10 | pv_tools.py |
| SDG&E daily fee | $0.345/day | pv_tools.py |
| EV charger power | 7.2 kW | pv_tools.py |
| EV daily energy | 14 kWh/EV | pv_tools.py |

## 4.2 Core Functions

Irradiance shape: G(t)/G_ref = max(0, sin(π(h - h_sunrise)/(h_sunset - h_sunrise))) with h_sunset = 18 + 2·sin(2π(doy-80)/365). PV output: P_array(t) = N × P_panel × shape(t) × PR, normalised so annual sum = N × P_panel × GHI × PR. Dispatch: surplus → charge battery then export; deficit → discharge battery then import. Economics: Net_CAPEX = (PV + Battery) × 1.10 × 0.70; Savings_y = Trad_bill_y - Solar_bill_y - O&M - Inverter; with degradation (1-d)^(y-1) and tariff escalation (1+0.06)^(y-1).

## 4.3 Hardware Selection

Panel: select_panel(brand) — auto = best efficiency/cost; user = exact manufacturer match. Battery: select_battery(required_kwh) — min cost/kWh when required_kwh > 0.5; else None. Battery need: battery_kwh = nighttime_fraction × annual_kwh/365 when nighttime_fraction > 0.30.

# 5. Stochastic Scenario

Determinism: household load is seeded by hashlib.sha256(lat_lon). Same location always yields identical profile. Stochastic elements: location (coastal vs inland, urban vs suburban), solar adoption (5–35% prob), EV adoption (8–30% prob), multi-generational (10–25% prob), household characteristics (normal draws), ±3% hourly noise. All pseudo-random; fixed per (lat, lon).

# 6. Hierarchical Prompt Structure

prompt_builder.py assembles the prompt in order: (1) Feature summary (~2,500 chars), (2) Equipment catalog (9 panels, 5 batteries, constants, EV assumptions), (3) User inputs, (4) Pre-computed tool results (panel, brand, battery, load, tariff, roof layout, sizing, recommended/optimal scenarios, battery analysis), (5) Hard rules (11 anti-hallucination constraints), (6) Decision policy (copy instructions), (7) JSON schema, (8) Task. Truncation: if > 24,000 chars, features truncated first; tool results, rules, schema, task protected.

# 7. Constraints Enforced

Hard rules: (1) Numeric integrity — copy from TOOL RESULTS > FEATURES > CATALOG > USER INPUTS; no own arithmetic. (2) Scenario structure — exactly two scenarios. (3) Evidence — 5–12 entries. (4) Brand — use user preference when specified. (5) Roof — N ≤ max_panels_by_roof. (6) Tariff — use rates from TOOL RESULTS. (7) Output — valid JSON only. (8) Self-check — panels×Wp/1000 = kw_dc. (9) Battery — copy from BATTERY ANALYSIS. (10) Panel brand — copy from BRAND SELECTION. Decision policy: N_opt = min(N_100, N_roof), N_rec = min(N_70, N_budget, N_roof), budget_binding = True if N_budget < N_70.

# 8. Equipment Catalogs

## 8.1 Solar Panels (9)

| Manufacturer | Model | Wp | Efficiency | $/Wp | Degrad/yr |
| --- | --- | --- | --- | --- | --- |
| REC Group | Alpha Pure | 405 | 22.6% | 2.85 | 0.005 |
| JA Solar | DeepBlue | 395 | 21.5% | 2.90 | 0.006 |
| Trina Solar | Vertex S | 400 | 21.8% | 2.90 | 0.006 |
| Canadian Solar | TOPHiKu7 | 420 | 22.5% | 3.10 | 0.005 |
| Silfab Solar | Prime | 410 | 22.1% | 3.15 | 0.005 |
| Jinko Solar | Tiger Neo | 440 | 23.8% | 3.20 | 0.005 |
| LONGi Solar | Hi-MO 6 | 435 | 23.3% | 3.35 | 0.005 |
| Maxeon Solar | Maxeon 7 | 430 | 22.8% | 3.50 | 0.004 |
| Aiko Solar | Neostar 2P | 460 | 24.3% | 3.75 | 0.004 |

## 8.2 Batteries (5)

| Manufacturer | Model | kWh | Cost | RTE |
| --- | --- | --- | --- | --- |
| Tesla | Powerwall 3 | 13.5 | $11,500 | 97.5% |
| Enphase | IQ Battery 5P | 5.0 | $6,000 | 96.0% |
| Generac | PWRcell M6 | 9.0 | $10,000 | 96.5% |
| SolarEdge | Home Battery 48V | 9.7 | $9,500 | 94.5% |
| Panasonic | EverVolt H Series | 17.1 | $15,000 | 97.0% |

# 9. Two Challenges and Solutions

## Challenge 1: LLM Hallucination of Financial Numbers

Problem: LLMs invent or round numbers. Solution: Pre-compute all numbers in pv_tools.run_all_tools() and pass them to the LLM. The LLM's job is to copy and format, not calculate. Hard rules and decision policy explicitly forbid arithmetic and require copying from TOOL RESULTS.

## Challenge 2: Household Load Without Real Data

Problem: No per-household metered data for arbitrary locations. Solution: Use EIA regional load as base, downscale by SDGE customer count, apply nine location-based factors. SHA-256 seeded RNG ensures same (lat, lon) + household params always yields identical profile. Deterministic, reproducible, location-specific household profiles.

# 10. Core Objects and Data Flow

Core objects: user_inputs (lat, lon, num_evs, num_people, num_daytime_occupants, budget_usd, roof_length_m, roof_breadth_m, rate_plan, panel_brand), tool_results (panel, brand, battery, load, tariff, roof layout, sizing, recommended/optimal scenarios, battery analysis), recommendation (optimal, recommended, battery_recommendation, panel_brand_recommendation, evidence).

Flow: User inputs → extraction → features → PV tools → prompt → LLM → validation → report.

— This report is based on the SolarInvest codebase: config.yaml, pv_tools.py, feature_engineering.py, prompt_builder.py, data_extractor.py, weather_fetcher.py, household_generator.py, and the README documentation.